Lab Experiment: Image Sharpening Using Laplacian Mask

Experiment No: 21

Aim

To perform sharpening of an image using a Laplacian mask implemented with diagonal neighbors.

Laplacian Mask

The given Laplacian mask is:

This mask uses 8-neighbor connectivity, including diagonal neighbors.

Tools Required

- Python 3.x

- OpenCV (cv2)

- NumPy

- VS Code / PyCharm / Jupyter Notebook

- Input image (image.jpg)

Procedure

- Install OpenCV and NumPy:

Bash

pip install opencv-python numpy

- Read the input image using OpenCV.

- Convert the image to grayscale.

- Define the Laplacian mask with diagonal neighbors.

- Apply the mask to the image using cv2.filter2D().

- Subtract the Laplacian result from the original image to obtain the sharpened image.

- Display the original, Laplacian, and sharpened images.

Code

import cv2

import numpy as np

# Read the image

image = cv2.imread("image.jpg")

# Check if image is loaded

if image is None:

print("Error: Image not found!")

exit(

# Convert image to grayscale

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

# Laplacian mask with diagonal neighbors

mask = np.array([

[1,  1,  1],

[1, -8,  1],

[1,  1,  1]

], dtype=np.float32)

# Apply Laplacian mask

laplacian = cv2.filter2D(gray, cv2.CV_32F, mask)

# Sharpen the image

sharpened = gray.astype(np.float32) - laplacian

# Convert values to valid 8-bit range

sharpened = np.clip(sharpened, 0, 255).astype(np.uint8)

# Display images

cv2.imshow("Original Image", gray)

cv2.imshow("Laplacian Image", cv2.convertScaleAbs(laplacian))

cv2.imshow("Sharpened Image", sharpened)

cv2.waitKey(0)

cv2.destroyAllWindows()

Output

Result

The image was successfully sharpened using the Laplacian mask with diagonal neighbors. The mask

was applied successfully, and the resulting image showed enhanced edges and details.